## Test Plan & Results

## 1. Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Motor Rotation | Press pushbutton | Motor runs smoothly for 10 seconds then stops |
| Buzzer Trigger | Press limit switch on prop | “Pew pew” sound plays correctly |
| Website Responsiveness | Switch themes multiple times | No lag or crash |
| System Stability | Run system continuously for long duration | No overheating or failure |
| Structural Stability | Test physical setup with all components | Frame holds motor and components safely |

## 2. User Testing

Users:

- One Classmate

- Team members

Tasks Given:

- Connect to the WiFi and open the website

- Select a theme

- Stand in front of the booth and use the prop

- Trigger the buzzer

- Experience the full 10-second interaction

## 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| Motor Rotation | Motor worked , then didn’t work, then worked again. | Finally Pass |
| Buzzer Trigger | Sound worked reliably | Pass |
| Website Responsiveness | Smooth most of the time, stopped working day before exhibition, worked with one neopixel, but was fixed later | Pass |
| System Stability | Stable after fixes | Pass |
| Structural Stability | Early design failed, final design stable | Fail → Pass |

## 4. Observations

- The prop interaction (buzzer) was the most engaging element

- Users enjoyed changing themes and seeing instant LED response

Pain Points:

- Initial confusion about when the motor starts (operator-controlled)

- Slight delay when loading website first time

- Early setup felt unstable and unsafe

## 5. Failures from Initial 360° Photobooth Idea

- DC motor extension was not 3D printed in time
→ Could not properly mount rotating mechanism

- Roof structure was too flimsy
→ Could not support weight of motor + arm + mobile

- Motor mounting was unstable
→ Structure couldn’t hold vibration during rotation

- Neo Pixel failure
→ Only 2 out of 8 LEDs worked (borrowed components were unreliable)

These failures led to:

- Downsizing to a compact backdrop

- Better structural control

- More reliable component usage

- Changing the 360 revolving camera idea.