# Software Documentation

## 1. Software Overview

The software controls the entire system by managing user input, lighting effects, motor movement, and sound output.

It:

- Hosts a web interface for selecting LED themes

- Controls Neo Pixel LEDs based on user selection

- Handles motor timing and rotation

- Triggers buzzer sound when the prop is used

- Runs all processes simultaneously using a continuous loop

Platforms Used:

- Micro Python (for ESP32 programming)

- Web Interface (HTML, CSS, JavaScript) hosted on ESP32

## 2. System Logic (Flow in Steps)

- ESP32 starts and creates a WiFi access point

- User connects and opens the web interface

- User selects a color theme

- ESP32 receives request and updates LED rings

- Operator presses pushbutton

- ESP32 starts motor rotation (10 seconds)

- User presses limit switch on prop

- ESP32 triggers buzzer sound pattern

- Motor automatically stops after 10 seconds

- System continues running and waits for next interaction

## 3. Flowchart (Logic)

## 4. App / Web Flow

### Screen Flow

- User connects to WiFi → Opens webpage

- Main screen shows theme selection cards

- Selected theme is highlighted

### Button Actions

- Clicking a theme card:

  - Sends request: /theme/<theme_name>

  - Updates LED colors instantly

### Data Flow

- Website → sends theme request → ESP32

- ESP32 → processes request → updates LEDs